# SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

MixerKing – Nicotine mixer for creating nicotine bases and e-liquids containing nicotine

REACH Registration Number:	01-2120066934-47-

CAS No:	54-11-5

Index No:	614-001-00-4

EC No:	200-193-3

1.2. Relevant identified uses of the substance or mixture and uses advised against

## Use of the substance/mixture

Use in liquids fillers for electronic cigarettes

Further information: refer to exposure scenarios attached to this safety data sheet.

## Uses advised against

Any non-intended use.

1.3. Details of the supplier of the safety data sheet

| Company name: | PPH „Agnet” Jacek Król |
| --- | --- |
| Street: | Pojezierska 32 |
| Place: | 92-328 Łódź Poland |
| Telephone: | +48 508 934 319 |
| Contact person: | Jacek Król |
| e-mail: | kingecigar@gmail.com |
| 1.4. Emergency telephone | +48 508 934 319 (Mo-Fr 6:00-14:00) |

number:

# SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Regulation (EC) No. 1272/2008 Hazard categories:

Acute toxicity: Acute Tox. 2

Acute toxicity: Acute Tox. 2

Acute toxicity: Acute Tox. 2

Skin corrosion/irritation: Skin Irrit. 2

Serious eye damage/eye irritation: Eye Dam. 1 Hazardous to the aquatic environment: Aquatic Chronic 2

Hazard Statements:

Fatal if inhaled.

Fatal in contact with skin.

Fatal if swallowed.

Causes skin irritation.

Causes serious eye damage.

Toxic to aquatic life with long lasting effects.

2.2. Label elements

## Regulation (EC) No. 1272/2008 Signal word:	Danger

Pictograms:

## Hazard statements

H300+H310+H330	Fatal if swallowed, in contact with skin or if inhaled.

H315	Causes skin irritation.

H318	Causes serious eye damage.

H411	Toxic to aquatic life with long lasting effects.

## Precautionary statements

P260	Do not breathe dust/fume/gas/mist/vapours/spray.

P273	Avoid release to the environment.

P280	Wear protective gloves/protective clothing/eye protection/face protection.

P301+P310	IF SWALLOWED: Immediately call a POISON CENTER/doctor.

P305+P351+P338	IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P403+P233	Store in a well-ventilated place. Keep container tightly closed.

2.3. Other hazards

This substance does not meet the PBT/vPvB criteria of REACH, Annex XIII.

# SECTION 3: Composition/information on ingredients

3.1. Substances

## Hazardous components

| CAS No | Chemical name |  |  | Quantity |
| --- | --- | --- | --- | --- |
|  | EC No | Index No | REACH No |  |
|  | GHS Classification |  |  |  |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |  | 99-100 % |
|  | 200-193-3 | 614-001-00-4 | 01-2120066934-47- |  |
|  | Acute Tox. 2, Acute Tox. 2, Acute Tox. 2, Skin Irrit. 2, Eye Dam. 1, Aquatic Chronic 2; H330 H310 H300 H315 H318 H411 |  |  |  |

Full text of H and EUH statements: see section 16.

## Further Information

Product does not contain listed SVHC substances > 0,1 % according to Regulation (EC) No. 1907/2006 Article 59 (REACH)

# SECTION 4: First aid measures

4.1. Description of first aid measures

## General information

Self-protection of the first aider.

In case of accident or unwellness, seek medical advice immediately (show directions for use or safety data sheet if possible).

## After inhalation

In case of accident by inhalation: remove casualty to fresh air and keep at rest. In case of breathing difficulties administer oxygen. In case of irregular breathing or respiratory arrest provide artificial respiration. Call a physician immediately.

After contact with skin Wash with plenty of water.

In case of skin contact, wash immediately with 3%-acetic acid and plenty of water.  Immediately remove any wetted clothing, shoes or stockings. Call a physician immediately.

## After contact with eyes

In case of contact with eyes, rinse immediately with plenty of flowing water for 10 to 15 minutes holding eyelids apart. Subsequently consult an ophthalmologist.

## After ingestion

If accidentally swallowed rinse the mouth with plenty of water (only if the person is conscious) and obtain immediate medical attention. Induce vomiting when the affected person is not unconscious. Never give anything by mouth to an unconscious person or a person with cramps.

4.2. Most important symptoms and effects, both acute and delayed

Dizziness. excitation. spasms. vomiting. Heart circulatory collapse. Headache.  unconsciousness.

4.3. Indication of any immediate medical attention and special treatment needed First Aid, decontamination, treatment of symptoms.

# SECTION 5: Firefighting measures

5.1. Extinguishing media

## Suitable extinguishing media

Suitable extinguishing media: Foam. Extinguishing powder. Carbon dioxide. Water spray.

Unsuitable extinguishing media High power water jet.

5.2. Special hazards arising from the substance or mixture Can be released in case of fire:

Carbon dioxide (CO2). Carbon monoxide Nitrogen oxides (NOx).

5.3. Advice for firefighters

In case of fire: Wear self-contained breathing apparatus.

## Additional information

Use water spray jet to protect personnel and to cool endangered containers. Contaminated fire-fighting water must be collected separately. Do not allow to enter into surface water or drains. Co-ordinate fire-fighting measures to the fire surroundings.

# SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

Remove persons to safety. Provide adequate ventilation. Use appropriate respiratory protection. Do not breathe gas/fumes/vapour/spray. Wear personal protection equipment.

Use a positive-pressure air-supplied respirator if there is any potential for an uncontrolled release, exposure levels are not known, or any other circumstances where air-purifying respirators may not provide adequate protection.

6.2. Environmental precautions

Do not allow to enter into surface water or drains. Eliminate leaks immediately. Prevent spread over a wide area (e.g. by containment or oil barriers). Do not allow to enter into soil/subsoil. If required, notify relevant authorities according to all applicable regulations.

6.3. Methods and material for containment and cleaning up Wear personal protection equipment.

Absorb with liquid-binding material (e.g. sand, diatomaceous earth, acid- or universal binding agents). Treat the recovered material as prescribed in the section on waste disposal. Clear contaminated areas thoroughly.

6.4. Reference to other sections refer to chapter 8

# SECTION 7: Handling and storage

7.1. Precautions for safe handling

## Advice on safe handling

Wear suitable protective clothing. ( See section 8. )

Advice on protection against fire and explosion Usual measures for fire prevention.

## Further information on handling

General protection and hygiene measures: refer to chapter 8 Read label before use.

Keep away from heat.

7.2. Conditions for safe storage, including any incompatibilities

## Requirements for storage rooms and vessels

Keep container tightly closed. Keep only in the original container in a cool, well-ventilated place. Keep locked up.

Recommended storage temperature: < 20 °C

## Hints on joint storage

Do not store together with: Flammable liquids. Flammable solids. Food and fodder  Gas.   Pyrophoric substances. Substances and mixtures which, in contact with water, emit flammable gases Oxidizing solids Oxidizing liquids. Radioactive substances. Infectious substances.

## Further information on storage conditions

Protect against: frost. UV-radiation/sunlight. heat. Cold  Humidity

7.3. Specific end use(s) See section 1.

# SECTION 8: Exposure controls/personal protection

8.1. Control parameters

## Exposure limits (EH40)

| CAS No | Substance | ppm | mg/m³ | fibres/ml | Category | Origin |
| --- | --- | --- | --- | --- | --- | --- |
| 54-11-5 | Nicotine | - | 0.5 |  | TWA (8 h) | WEL |
|  |  | - | 1.5 |  | STEL (15 min) | WEL |

## DNEL/DMEL values

| CAS No | Substance |  |  |  |
| --- | --- | --- | --- | --- |
| DNEL type |  | Exposure route | Effect | Value |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |  |  |
| Worker DNEL, long-term |  | inhalation | systemic | 0,0313 mg/m³ |
| Worker DNEL, acute |  | inhalation | systemic | 8,6 mg/m³ |
| Worker DNEL, long-term |  | dermal | systemic | 0,00443 mg/kg bw/day |
| Worker DNEL, acute |  | dermal | systemic | 0,84 mg/kg bw/day |
| Worker DNEL, acute |  | dermal | local | 0,2 mg/cm² |
| Consumer DNEL, long-term |  | inhalation | systemic | 0,1555 mg/m³ |
| Consumer DNEL, acute |  | inhalation | systemic | 9,6 mg/m³ |
| Consumer DNEL, long-term |  | dermal | systemic | 0,4472 mg/kg bw/day |
| Consumer DNEL, acute |  | dermal | systemic | 2,2 mg/kg bw/day |
| Consumer DNEL, acute |  | dermal | local | 0,2 mg/cm² |
| Consumer DNEL, long-term |  | oral | systemic | 0,0511 mg/kg bw/day |
| Consumer DNEL, acute |  | oral | systemic | 0,61 mg/kg bw/day |

## PNEC values

| CAS No | Substance |  |
| --- | --- | --- |
| Environmental compartment |  | Value |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |
| Freshwater |  | 0,0004 mg/l |
| Freshwater (intermittent releases) |  | 2,7 mg/l |
| Marine water |  | 0,00004 mg/l |
| Marine water (intermittent releases) |  | 0,000065 mg/kg |
| Freshwater sediment |  | 0,00065 mg/kg |
| Soil |  | 0,000321 mg/kg |

8.2. Exposure controls

## Appropriate engineering controls

If handled uncovered, arrangements with local exhaust ventilation have to be used.

When workers are facing concentrations above the exposure limit they must use appropriate certified respirators.

## Protective and hygiene measures

Always close containers tightly after the removal of product. When using do not eat, drink, smoke, sniff. Wash hands before breaks and after work. Remove contaminated clothing immediatley and dispose off safely. Wash contaminated clothing prior to re-use. Used working clothes should not be worn outside the work area. Street clothing should be stored seperately from work clothing.

Avoid contact during pregnancy/while nursing.

## Eye/face protection

Suitable eye protection: Tightly sealed safety glasses. DIN EN 166

Hand protection Wear suitable gloves.

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Suitable material:

Butyl rubber. Thickness of glove material: (0,5 mm) FKM (fluororubber). Thickness of glove material: (0,4 mm) Breakthrough time ~480 min.

penetration time (maximum wearing period): ~160 min.

In the case of wanting to use the gloves again, clean them before taking off and air them well.

Skin protection

Suitable protective clothing: Lab apron.

## Respiratory protection

With correct and proper use, and under normal conditions, breathing protection is not required. Respiratory protection necessary at: aerosol or mist generation.

Exceeding exposure limit values

Suitable respiratory protective equipment: Combination filtering device (EN 14387)  Type: A-(P3)

The filter class must be suitable for the maximum contaminant concentration (gas/vapour/aerosol/particulates) that may arise when handling the product. If the concentration is exceeded, self-contained breathing apparatus must be used.

Environmental exposure controls

This material and its container must be disposed of in a safe way.

# SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Physical state:	liquid, viscous

Colour:	colourless or brownish Odour:	characteristic

pH-Value:	10,2

## Changes in the physical state

Melting point:	-79 °C

Initial boiling point and boiling range:	246 °C

Sublimation point:	No information available.

Softening point:	No information available. Pour point:	No information available.

Flash point:	101 °C

Sustaining combustion:	No data available

## Flammability

Solid:	No information available. Gas:	No information available.

Explosive properties No information available.

Lower explosion limits:	0,7 vol. %

Upper explosion limits:	4,0 vol. % Ignition temperature:	240 °C

## Auto-ignition temperature

Solid:	No information available.

Gas:	No information available.

Decomposition temperature:	No information available.

Oxidizing properties No information available.

Vapour pressure:	0,056 hPa

(at 20 °C)

Vapour pressure:	No information available.

(at 50 °C)

| Density (at 20 °C): | 1,01 g/cm³ |
| --- | --- |
| Bulk density: | No information available. |
| Water solubility: Solubility in other solvents Ethanol. | miscible. |
| Partition coefficient: | No information available. |
| Viscosity / dynamic: | No information available. |
| Viscosity / kinematic: | No information available. |
| Flow time: | No information available. |
| Vapour density: | No information available. |
| Evaporation rate: | No information available. |
| Solvent separation test: | No information available. |
| Solvent content: | not determined |

9.2. Other information

| Solid content: No information available. | not determined |
| --- | --- |
| SECTION 10: Stability and reactivity |  |

10.1. Reactivity

No information available.

10.2. Chemical stability

The product is stable under storage at normal ambient temperatures.

10.3. Possibility of hazardous reactions No information available.

10.4. Conditions to avoid heat. Oxidizing agents.

10.5. Incompatible materials

Materials to avoid: Oxidizing agents, strong. Reducing agents, strong.

10.6. Hazardous decomposition products

Can be released in case of fire: Carbon dioxide (CO2). Carbon monoxide Nitrogen oxides (NOx).

Further information

No information available.

# SECTION 11: Toxicological information

11.1. Information on toxicological effects

Toxicocinetics, metabolism and distribution No information available.

## Acute toxicity

Fatal if swallowed.

Fatal in contact with skin.

Fatal if inhaled.

ATE oral  (nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine) = 5 mg/kg*

ATE dermal (nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine) = 70 mg/kg* ATE Inhalation (nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine) = 0,19 mg/l* Literature information:

*Committee for Risk Assessment RAC Opinion proposing harmonised classification and labelling at EU level of

Nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine, 09/2015

| CAS No | Chemical name |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Exposure route | Dose | Species | Source | Method |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |  |  |  |
|  | oral | ATE 5 mg/kg |  |  |  |
|  | dermal | ATE 70 mg/kg |  |  |  |
|  | inhalation (4 h) vapour | LC50 <2 mg/l |  |  |  |
|  | inhalation aerosol | ATE 0,19 mg/kg |  |  |  |

## Irritation and corrosivity

Causes skin irritation.

Causes serious eye damage.

nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine:

Irritant effect on the respiratory tract: slightly irritant but not relevant for classification. Irritant effect on the eye:

Method: OECD Guideline 405

Species: Rabbit

Result: positive.

Irritant effect on the skin:

Method: OECD Guideline 402 (Acute Dermal Toxicity)

Species: Rabbit Result: positive.

literature infomation: ECHA Dossier

## Sensitising effects

Based on available data, the classification criteria are not met. nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine:

Skin sensitisation:

Method:  OECD Guideline 429 (Skin Sensitisation: Local Lymph Node Assay)

Species: Mouse Result: negative.  literature infomation: ECHA Dossier

Carcinogenic/mutagenic/toxic effects for reproduction Based on available data, the classification criteria are not met. nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine:

In-vitro mutagenicity: Ames test negative. (Salmonella typhimurium)

[BRAMS,A, BUCHET,JP, CRUTZEN-FAYT,MC, DE MEESTER,C, LAUWERYS,R AND LEONARD,A; A

COMPARATIVE STUDY, WITH 40 CHEMICALS, OF THE EFFICIENCY OF THE SALMONELLA ASSAY AND THE SOS CHROMOTEST (KIT PROCEDURE); TOXICOL. LETT. 38(1-2):123-133, 1987]

Reproductive toxicity:

Method:: OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction /

Developmental Toxicity Screening Test); Species: Rat; Exposure route: Inhalation, Result: NOEL = 20  mg/m³,

NOAEL> = 20  mg/m³; Literature information: ECHA Dossier

## STOT-single exposure

Based on available data, the classification criteria are not met.

No information available.

## STOT-repeated exposure

Based on available data, the classification criteria are not met. nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine: NOAEL: 1,25 mg/kg (EFSA, 2009) Subacute inhalative toxicity:

Method: OECD Guideline 422 (Combined Repeated Dose Toxicity Study with the Reproduction /

Developmental Toxicity Screening Test)

Species: Rat

Results: LOAEC = 10 mg/m³

Literature information: ECHA Dossier

## Aspiration hazard

Based on available data, the classification criteria are not met.

No information available.

Specific effects in experiment on an animal No information available.

Additional information on tests No information available.

Practical experience

Observations relevant to classification No information available.

Other observations No information available.

Further information

No information available.

# SECTION 12: Ecological information

12.1. Toxicity

| CAS No | Chemical name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Aquatic toxicity | Dose |  | [h] ¦ [d] | Species | Source | Method |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-m | ethylpyrrolidin-2-yl]pyri |  | dine |  |  |  |
|  | Acute fish toxicity | LC50 | >3 mg/l | 96 h | Oncorhynchus mykiss | ECHA Dossier | OECD Guideline 203 |
|  | Acute algae toxicity | ErC50 | 37 mg/l | 72 h | Desmodesmus subspicatus ECHA Dossier | ECHA Dossier | EU Method C.3 |
|  | Acute crustacea toxicity | EC50 | 3 mg/l | 48 h | Daphnia magna | ECHA Dossier | OECD Guideline 202 |
|  | Crustacea toxicity | NOEC mg/l | 0,012 | 16 d | Daphnia pulex | ECHA Dossier | OECD Guideline 211 |

12.2. Persistence and degradability

| CAS No | Chemical name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Method |  | Value | d | Source |  |
|  | Evaluation |  |  |  |  |  |
| 54-11-5 | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |  |  |  |  |
|  | OECD 301B / ISO 9439 / EEC 92/69 annex V, C.4-C |  | 71% | 8 | ECHA Dossier |  |
|  | Easily biodegradable (concerning to the criteria of the OECD) |  |  |  |  |  |
| 12.3. Bioaccumulative potential No indication of bioaccumulation potential. Partition coefficient n-octanol/water |  |  |  |  |  |  |
| CAS No |  | Chemical name |  |  |  | Log Pow |
| 54-11-5 |  | nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine |  |  |  | 1,17 (pH> 12) |

12.4. Mobility in soil

No information available.

12.5. Results of PBT and vPvB assessment

This substance does not meet the PBT/vPvB criteria of REACH, Annex XIII.

12.6. Other adverse effects No information available.

Further information

Do not allow uncontrolled discharge of product into the environment.

# SECTION 13: Disposal considerations

13.1. Waste treatment methods

## Advice on disposal

Dispose of waste according to applicable legislation. Consult the local waste disposal expert about waste disposal. Non-contaminated packages may be recycled. The allocation of waste identity numbers/waste descriptions must be carried out according to the EEC, specific to the industry and process.

Control report for waste code/ waste marking according to (EWC) European Waste Catalogue:

## Waste disposal number of waste from residues/unused products

160305	WASTES NOT OTHERWISE SPECIFIED IN THE LIST; off-specification batches and unused products; organic wastes containing hazardous substances; hazardous waste

## Waste disposal number of used product

160305	WASTES NOT OTHERWISE SPECIFIED IN THE LIST; off-specification batches and unused products; organic wastes containing hazardous substances; hazardous waste

## Waste disposal number of contaminated packaging

150110	WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED; packaging (including separately collected municipal packaging waste); packaging containing residues of or contaminated by hazardous substances; hazardous waste

Contaminated packaging

Handle contaminated packages in the same way as the substance itself.

# SECTION 14: Transport information

## Land transport (ADR/RID)

| 14.1. UN number: | UN 1654 |
| --- | --- |
| 14.2. UN proper shipping name: | NICOTINE |
| 14.3. Transport hazard class(es): | 6.1 |
| 14.4. Packing group: | II |

Hazard label:	6.1

| Classification code: | T1 |
| --- | --- |
| Special Provisions: | 274 335 375 601 |
| Limited quantity: | 100 mL |
| Excepted quantity: | E4 |
| Transport category: | 2 |
| Hazard No: | 60 |
| Tunnel restriction code: | D/E |

## Other applicable information (land transport) Temperature, Transport information: 30 °C Inland waterways transport (ADN)

| 14.1. UN number: | UN 1654 |
| --- | --- |
| 14.2. UN proper shipping name: | NICOTINE |
| 14.3. Transport hazard class(es): | 6.1 |
| 14.4. Packing group: | II |
| Hazard label: | 6.1 |

| Classification code: | T1 |
| --- | --- |
| Special Provisions: | 802 |
| Limited quantity: | 100 mL |
| Excepted quantity: | E4 |

## Other applicable information (inland waterways transport) Temperature, Transport information: 30 °C Marine transport (IMDG)

| 14.1. UN number: | UN 1654 |
| --- | --- |
| 14.2. UN proper shipping name: | NICOTINE |
| 14.3. Transport hazard class(es): | 6.1 |
| 14.4. Packing group: | II |
| Hazard label: | 6.1 |

| Marine pollutant: | YES |
| --- | --- |
| Special Provisions: | - |
| Limited quantity: | 100 mL |
| Excepted quantity: | E4 |
| EmS: | F-A, S-A |

## Other applicable information (marine transport) Temperature, Transport information: 30 °C Air transport (ICAO-TI/IATA-DGR)

| 14.1. UN number: | UN 1654 |
| --- | --- |
| 14.2. UN proper shipping name: | NICOTINE |
| 14.3. Transport hazard class(es): | 6.1 |

| 14.4. Packing group: | II |
| --- | --- |
| Hazard label: | 6.1 |

| Special Provisions: | A97 A158 A197 |  |
| --- | --- | --- |
| Limited quantity Passenger: | 1 L |  |
| Passenger LQ: | Y641 |  |
| Excepted quantity: | E4 |  |
| IATA-packing instructions - Passenger: |  | 654 |
| IATA-max. quantity - Passenger: |  | 5 L |
| IATA-packing instructions - Cargo: |  | 662 |
| IATA-max. quantity - Cargo: |  | 60 L |

## Other applicable information (air transport)

Temperature, Transport information: 30 °C 14.5. Environmental hazards

ENVIRONMENTALLY HAZARDOUS: 	yes

Danger releasing substance:	nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine:

14.6. Special precautions for user Refer to section 6-8

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code not relevant

Other applicable information

No information available.

# SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

## EU regulatory information

2010/75/EU (VOC):	0%

2004/42/EC (VOC):	100 % (1010 g/l)

Information according to 2012/18/EU 	H2 ACUTE TOXIC (SEVESO III):

Additional information:	E2

## Additional information

This substance is hazardous in the sense of regulation (EC) No 1272/2008 (CLP). REACH 1907/2006 Appendix XVII, No (mixture): 3

## National regulatory information

| Employment restrictions: | Observe restrictions to employment for juvenils according to the 'juvenile work protection guideline' (94/33/EC). Observe employment restrictions under the Maternity Protection Directive (92/85/EEC) for expectant or nursing mothers. |
| --- | --- |
| Water contaminating class (D): | 3 - highly water contaminating |

15.2. Chemical safety assessment

For the following substances of this mixture a chemical safety assessment has been carried out:

nicotine (ISO); 3-[(2S)-1-methylpyrrolidin-2-yl]pyridine

# SECTION 16: Other information

## Changes

Rev. 1,0; 06.12.2013;  Initial release

Rev. 1,01; 28.01.2014

Rev. 1,02; 05.05.2014

Rev. 1,10; 26.06.2015; Changes in chapter: 1-16

Rev. 1,20; 02.06.2017; Changes in chapter: 1-16

Rev. 2,00; 06.12.2018; Changes in chapter: 2,3,8,11,12, 15, 16

Rev. 2,10; 18.10.2019; Changes in chapter: 9, 14

## Abbreviations and acronyms

ADR: Accord européen sur le transport des marchandises dangereuses par Route

AwSV: Verordnung über Anlagen zum Umgang mit wassergefährdenden Stoffen AGW: Arbeitsplatzgrenzwert

AVV: Abfallverzeichnisverordnung

CAS Chemical Abstracts Service

CLP: Classification, Labelling and Packaging of substances and mixtures DNEL: Derived No Effect Level d: day(s)

EAKV: Europäisches Abfallverzeichnis gemäß Entwurf Abfallverzeichnisverordnung

EINECS: European INventory of Existing Commercial chemical Substances

ELINCS: European LIst of Notified Chemical Substances

ECHA: European Chemicals Agency

EWC: European Waste Catalogue

IARC: INTERNATIONAL AGENCY FOR RESEARCH ON CANCER

IMDG: International Maritime Code for Dangerous Goods

IATA: International Air Transport Association

IATA-DGR: Dangerous Goods Regulations by the "International Air Transport Association" (IATA) ICAO: International Civil Aviation Organization

ICAO-TI: Technical Instructions by the "International Civil Aviation Organization" (ICAO)

GHS: Globally Harmonized System of Classification and Labelling of Chemicals GefStoffV: Gefahrstoffverordnung (Ordinance on Hazardous Substances, Germany) h: hour

LOAEL: Lowest observed adverse effect level

LOAEC: Lowest observed adverse effect concentration

LC50: Lethal concentration, 50 percent

LD50: Lethal dose, 50 percent

NOAEL: No observed adverse effect level

NOAEC: No observed adverse effect level

NLP: No-Longer Polymers

N/A: not applicable

OECD: Organisation for Economic Co-operation and Development

PNEC: predicted no effect concentration

PBT: Persistent bioaccumulative toxic

RID: Règlement international concernant le transport des marchandises dangereuses par chemin de fer (Regulations Concerning the International Transport of Dangerous Goods by Rail )

REACH: Registration, Evaluation, Authorisation of Chemicals

SVHC: substance of very high concern

TRGS Technische Regeln fuer Gefahrstoffe

UN: United Nations

VOC: Volatile Organic Compounds

VwVwS: Verwaltungsvorschrift wassergefaehrdender Stoffe

WGK: Wassergefaehrdungsklasse

## Relevant H and EUH statements (number and full text)

H300	Fatal if swallowed.

H300+H310+H330	Fatal if swallowed, in contact with skin or if inhaled.

H310	Fatal in contact with skin.

H315	Causes skin irritation.

H318	Causes serious eye damage.

H330	Fatal if inhaled.

H411	Toxic to aquatic life with long lasting effects.

## Further Information

Classification according to Regulation (EC) No 1272/2008 [CLP] - Classification procedure:

Health hazards: Calculation method.

Environmental hazards: Calculation method.

Physical hazards: On basis of test data and / or calculated and / or estimated.

The above information describes exclusively the safety requirements of the product and is based on our present-day knowledge. The information is intended to give you advice about the safe handling of the product named in this safety data sheet, for storage, processing, transport and disposal. The information cannot be transferred to other products. In the case of mixing the product with other products or in the case of processing, the information on this safety data sheet is not necessarily valid for the new made-up material.

## Identified uses

| No |  | Short title | LCS | SU | PC |  | PROC | ERC |  | AC | TF | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Formulation of nicotine salts and nicotine mixtures. | - | - | 0 |  | 1, 2, 3, 4, 5, 8a, 8b, 9, 15, 28 | 2, 3 |  | - | 84 |  |
| 2 |  | Industrial use of intermediates. | - | 9, 20 | 29 |  | 1, 2, 3, 8a, 8b, 9, 15, 28 | 6a |  | - | 77 |  |
| 3 |  | Consumer use (To use in electronic cigarettes.) | - | - | 0 |  | - | 8a, 8d |  | - | 84 |  |
| LCS: Life cycle stages |  |  |  |  | SU: Sectors of use |  |  |
| PC: Product categories |  |  |  |  | PROC: Process categories |  |  |
| ERC: Environmental release categories TF: Technical functions |  |  |  |  | AC: Article categories |  |  |